PSI CRIMPED STEEL FIBER MB

Steel macro-fiber with synthetic multifilament micro-fiber blend for slabs-on-grade, toppings, and shotcrete

{Note to Specifier: The paragraphs below are meant to be incorporated into Parts 1, 2, and 3 of a standard CSI 3 Part Format specification (normally 03 30 00), the General Structural Notes, or directly onto the plans. They must be carefully reviewed and edited by a qualified design professional to meet the requirements of the project and governing building codes. Coordinate with other specification sections and drawings.}

PART 1:	GENERAL

- 1._	PRE-CONCRETE MEETING

- Prior to the start of the concreting the General Contractor shall conduct a meeting to review the required methods and procedures to achieve the required concrete placement. The General Contractor shall send a meeting agenda to all attendees 20 days prior to the scheduled date of the meeting.

- The contractor shall require responsible representatives of every party concerned with the concreting work to attend the meeting, including but not limited to the following: Contractor’s Superintendent, Ready-Mix Company, Concrete Contractor, Testing Lab, Fiber Manufacturer, and Owner’s Representative.

- Minutes of the meeting shall be recorded, typed, and distributed by the General Contractor to all concerned parties, including but not limited to the Owner’s representative, the Architect, and all attendees within 5 days of the meeting.

PART 2:	PRODUCTS

2._	STEEL MACRO-FIBER WITH SYNTHETIC MULTIFILAMENT MICRO-FIBER REINFORCEMENT

A.	Steel Macro-Fiber with Synthetic Multifilament Micro-Fiber Blend Reinforcement: Low carbon, cold drawn steel wire fibers combined with 100% virgin multifilament polypropylene micro-synthetic fibers. Complying with ASTM C 1116/C 1116 M and ASTM A820. Steel fibers 1.5 inch (38 mm) length with a minimum tensile strength of 120,000 psi in combination with synthetic fibers blend of ½ inch and ¾ inch length (13 mm and 19 mm).

- 1.	Product:

- Euclid Chemical Company (The); PSI Crimped Steel Fiber MB; www.euclidchemical.com

- Fiber manufacturer shall have ISO 9001 certification.

2._	CONCRETE  MIXES

- A.	Use Steel Macro-Fiber with Synthetic Multifiliment Micro-Fiber Blend Reinforcement for slabs-on-grade, toppings, and shotcrete at rate of [Insert dosage provided to specifier by fiber manufacturer] pounds per cubic yard of concrete.

PART 3:	EXECUTION

3._	STEEL MACRO-FIBER WITH SYNTHETIC MULTIFILIMENT MICRO-FIBER REINFORCEMENT

- Steel Macro-Fiber with Synthetic Multifiliment Micro-Fiber Blend Reinforcement:  Add to concrete and mix for 4 – 5 minutes to provide uniform distribution.

- For broomed surfaces, broom once in one direction only.