PSI FIBERSTRAND N

Nylon Micro-Fiber

{Note to Specifier: The paragraphs below are meant to be incorporated into Parts 2 and 3 of a standard CSI 3 Part Format specification (normally 03 30 00), the General Structural Notes, or directly onto the plans. They must be carefully reviewed and edited by a qualified design professional to meet the requirements of the project and governing building codes. Coordinate with other specification sections and drawings.}

PART 2:	PRODUCTS

2.__	NYLON MICRO-FIBER

A.	Multifilament nylon fibers complying with ASTM C 1116/C 1116M 4.1.3. Available in 1/2 inch (13 mm),  ¾ inch (19 mm), and inch (39 mm).

1.	Product:

a)   Euclid Chemical Company (The); PSI Fiberstrand N;

www.euclidchemical.com.

- Fiber manufacturer shall have ISO 9001 certification.

2._	CONCRETE  MIXES

- A.	Use Nylon Micro-Fibers at rate of 1.0 pound per cubic yard of concrete.

PART 3:	EXECUTION

3._	NYLON MICRO-FIBER

- Nylon Micro-Fiber:  Add to concrete and mix for 3 – 5 minutes to provide uniform distribution.

- For broomed surfaces, broom once in one direction only.