EUCO CABLE GROUT PTX

High Performance Cable Grout

{Note to Specifier: This add-in specification component specifies Euclid Chemical Company, EUCO CABLE GROUT PTX high performance cable grout.  EUCO CABLE GROUT PTX is formulated to produce a pumpable, non-shrink, high strength grout. designed for critical corrosion protection of post tensioning cable ducts, anchorages and rods. EUCO CABLE GROUT PTX is extremely fluid and contains an expansive cementitious binder. EUCO CABLE GROUT PTX does not contain aggregate or filler. EUCO CABLE GROUT PTX exhibits thixotropic properties defined by Post Tensioning Institute (PTI) specifications and can be used to repair previously grouted cables. It remains pumpable for a minmum of 2 hours at 90o F (32o C) and can be placed at fluid and flowable consistencies.}

{Note to Specifier: The paragraphs below are meant to be incorporated into Parts 2 and 3 of a standard CSI 3 Part Format specification, project’s General Structural Notes or directly onto the plans. They must be carefully reviewed by a qualified design professional and edited to meet the particular requirements of the project at hand, assure compliance with any governing building codes, and coordinate with other specification sections and drawings.}

1.1	REFERENCES

A.	Post-Tensioning Institute PTI M55.1-12 Specification for Grouting of Post-Tensioned Structures

1.2	QUALITY REQUIREMENTS

A.	Manufacturer: ISO 9001 quality certified as primary manufacturer of specified products.

1.3	INFORMATIONAL SUBMITTALS

A.	Product List: List manufacturer name and product name for each product proposed for use as cementitious grout.

B.	Manufacturer Certificate: Indicating products listed on Contractor's Product List are compatible and suitable for the specified application.

PART 2:	PRODUCTS

2.1	MANUFACTURERS

A.	Manufacturer: Provide listed products of Euclid Chemical Co., Cleveland, OH; www.euclidchemical.com.

B.	Manufacturer Single Source: Provide cementitious grout products from a single qualified

manufacturer.

2.2	CEMENTITIOUS GROUT

A.	Basis of Design Product:

1.	“EUCO CABLE GROUT PTX” by The Euclid Chemical Company

2.	Flow rate, ASTM C939, modified:

a)	Fluid consistency, 1.5 gal./50 lb (5.7 L/22.7 kg)

1)	9-20 seconds, intial flow

2)	9-30 at 30 minutes, remixed for 30 seconds prior to retest

3.	Compressive Strength, ASTM C942:

a)	Fluid consistency, 1.5 gal./50 lb (5.7 L/22.7 kg)

1)	7 days: > 3000 psi (20.7 MPa)

2)	28 days: > 7000 psi (48.3 MPa)

4.	Plastic Expansion, ASTM C940:

a)	Fluid consistency, 1.5 gal./50 lb (5.7 L/22.7 kg)

1)	3 days: 0.0%  to 2.0% for up to 3 hours

5.	Setting time @70oF (21oC), ASTM C953:

a)	Initial set: 8 to 12 hours

6.	Wick induced bleed, ASTM C940 modified according to C.4.4.6.1 of the PTI Guide Specification

a)	5 minutes: 0.0%

b)	3 hours: 0.0%

7.	Shupak pressure bleed test

a)	@ 1.5 gal./50 lb (5.7 L/22.7 kg) mix water

b)	0.0% (5 minutes @ 100 psi (0.7 MPa))

c)	@ 1.7 gal./50 lb (6.4 L/22.7 kg) mix water

d)	0.0% (5 minutes @ 50 psi (0.3 MPa))

8.	Chloride permeability, ASTM C1202

a)	28 days, 30 V for 6hours:<2500 coulombs

PART 3:	EXECUTION

{Note to Specifier: This section covers surface preparation, form preparation, mixing, placement, curing, and cleanup for Euclid Chemical’s EUCO CABLE GROUT PTX. For complete current instructions and safety information, consult the Technical Data and Material Safety sheets posted at www.euclidchemical.com Note: The contractor and engineer are encouraged to consult and review the Euclid Chemical bulletin: “Cementitious Grout Application Guide”:

(http://euclidchemical.com/fileshare/ProductFiles/MiscProductFiles/grout_instructions.pdf.)

The document offers instructions detailing the general installation of Euclid Chemical manufactured cement-based grout products. Important: If the contractor is not familiar with standard grout placement techniques, a pre-job meeting is suggested to review the project details unique to the particular job. Contact your local Euclid Chemical representative for additional information.}

3.1	MIXING

A.	All grouting materials shall be 50°F to 80°F (10°C to 27°C) immediately prior to mixing.

B.	Mix grout in accordance with manufacturer’s written instructions. Mix small quantities with a drill and "jiffy" mixer. Use a paddle type mortar mixer for large jobs.

C.	Charge mixing vessel with appropriate amount of clean, potable water for batch size and then add dry grout. Do not exceed water content recommended by manufacturer. Control water content. Do not add admixtures, sand or cement to grout.

D.	Check flow.

3.2	PLACEMENT

3.3	Comply with PTI M55.1-12 Specification for Grouting of Post-Tensioned Structures

A.	Placed mixed grout immediately and continuously.

3.4	CURING

A.	Cure grout in accordance with manufacturer’s written instructions.

3.5	CLEAN UP

A.	Tools and mixer may be cleaned with water.