VANDEX SUPER/SUPER WHITE – Dry Shake Application

Crystalline Waterproofing Coating for Positive and Negative Side Waterproofing

{Note to Specifier: The paragraphs below are meant to be incorporated into Parts 1, 2, and 3 of a standard CSI 3 Part Format specification, the General Structural Notes, or directly onto the plans. They must be carefully reviewed and edited by a qualified design professional to meet the requirements of the project and governing building codes. Coordinate with other specification sections and drawings.}

PART 1:	GENERAL

{Note to Specifier: Insert the following paragraph and sub paragraphs as required for your project. Euclid’s recommended products are shown in italics. More info can be found on these products at www.euclidchemical.com or by clicking on the product links.}

1.01 	RELATED WORK:

A. 	Joint Fillers – Eucolastic, Tammsflex, Dural 340, Qwikjoint UVR

B. 	Concrete Repair:

1. 	Vertical and Overhead: Euco V-100, Tamms Structural Mortar

2. 	Horizontal: Express Repair, VersaSpeed

3. 	Form and Pour: Eucocrete

C. 	Crack Repair/Injection: Dural 452 LV, Dural Fast Set Epoxy Gel

D. 	Bonding Agents:  Duralprep A.C., Dural 452 MV

E. 	Waterproofing/Dampproofing : Tamoseal, Vandex Super, Hey’Di K-11, Vandex BB75

F. 	Architectural Coatings: Tammscoat, Tammolastic

G. 	Anti-Graffiti Coatings:  AG 100, AG-400

H. 	Traffic Deck Coatings: Tammsdeck, Flexdeck

I. 	Decorative Floor Coatings: Duraltex

J. 	Epoxy Chemical Resistant Coatings: Duralkote 240, Duralkote 500, Duraltex 1705/07, Duraltex 1805/07

K. 	Penetrating Water Repellents:

1. 	Horizontal and Vertical: Baracade WB 244, Baracade 100C, Baracade Silane 40 IPA

2. 	Vertical: Chemstop WB Regular/Heavy Duty

L. 	Penetrating Epoxy Sealer: Euco #512 VOX Epoxy Sealer

M. 	Cathodic Protection: Sentinel Galvanic Anodes

1.02	QUALITY ASSURANCE

A.	Installer Qualifications:  Experienced installer who has successfully completed 5 jobs of similar size and scope in the prior 5 years.

1.03	MOCKUP

A.	Cast 6’ by 6’ slab and wall mockups to demonstrate typical joints, surface finish, texture, color, and standard of workmanship.

B.	Notify Engineer at least 7 days in advance of date and time mockup will be constructed.

C.	Obtain approval of mockup by owner’s representative before starting construction.

D.	Maintain approved mockup in undisturbed condition throughout construction as standard for completed pavement.

E.	Demolish and remove mockup from site at completion of Work unless approved by owner’s representative to become part of completed Work.

1.04	PRE-APPLICATION MEETING

A.	Prior to the start of concrete placement Contractor shall conduct a meeting to review the required methods and procedures to achieve the required finish. Contractor shall send a meeting agenda to all attendees 20 days prior to the scheduled date of the meeting.

B.	The Contractor shall require responsible representatives of every party concerned with the concreting work to attend the meeting, including but not limited to the following: Contractor’s superintendent, ready-mix company, testing lab, waterproofing applicator, and owner’s representative.

C.	Minutes of the meeting shall be recorded, typed, and distributed by the Contractor to all concerned parties, including but not limited to the owner’s representative and all attendees within 5 days of the meeting.

PART 2:	PRODUCTS

2.01	WATERPROOFING

A.	NSF/ANSI Standard 61 approved, VOC compliant, breathable, crystalline waterproofing that can be applied by brush or spray. Crystalline waterproofing shall have minimum compressive strength of 3,500 psi at 28 days per ASTM C 109, minimum flexural strength of 900 psi at 28 days per ASTM C 348, minimum freeze/thaw resistance after 300 cycles of 98% per ASTM C 666, and be tested in accordance with CRD C 48-92.

1.	Product:

a)   Euclid Chemical Company (The); Vandex Super/Super White,

www.euclidchemical.com

B.	To ensure compatibility, all products shall be from the same manufacturer, who shall have ISO 9001 certification.

PART 3:	EXECUTION

3.01	DRY SHAKE WATERPROOFING APPLICATION

A.	Place and screed concrete as specified in Section <<insert section>>. Once the concrete has reached initial set and the bleed water has disappeared, use a power trowel with float shoes to open the surface of the concrete. Broadcast dry crystalline waterproofing powder over the surface at the [0.25 lbs per square foot][specified amount] with a mesh sieve or other mechanical means. The material must then be worked into the surface with float shoes prior to the start of final finishing procedures. B.

3.02	CURING

A.	Wet cure for minimum 7 days per manufacturer’s written recommendations.

END OF SECTION