EUCON INTEGRAL ARC

Alkali Silica Reactivity (ASR) Control Admixture

{Note to Specifier: The paragraphs below are meant to be incorporated into Part 2 of a standard CSI 3 Part Format specification (normally 03 30 00), the General Structural Notes, or directly onto the plans. They must be carefully reviewed and edited by a qualified design professional to meet the requirements of the project and governing building codes. Coordinate with other specification sections and drawings.}

PART 2:	PRODUCTS

2.__	ADMIXTURES

A.	Alkali Silica Reactivity Control Admixture:  Lithium nitrate admixture meeting ASTM C 494 Type S and ANSI/NSF STD 61.

1.	Product:

a)	Euclid Chemical Company (The); Eucon Integral ARC, www.euclidchemical.com

2.__	MIXES

A.	Use Alkali Silica Reactivity Control Admixture unless ready mix supplier submits testing per ASTM C 1260, ASTM C 1567, or ASTM C 1293 showing that aggregates are non-reactive.

2.__	MANUFACTURER

A.	Manufacturer shall have ISO 9001 Quality Certification and to ensure compatibility all admixtures shall be from the same manufacturer.